Smart Parking Garage Gate System

Test Case Report  —  CSE411 / CSE323  Spring 2026

Test Results Summary

| TC# | Name | Result |
| --- | --- | --- |
| TC-01 | Manual OPEN – stops on release | PASS |
| TC-02 | Manual CLOSE – stops on release | PASS |
| TC-03 | Manual from STOPPED_MIDWAY | PASS |
| TC-04 | Auto OPEN from IDLE_CLOSED | PASS |
| TC-05 | Auto CLOSE from IDLE_OPEN | PASS |
| TC-06 | Auto mode from STOPPED_MIDWAY | PASS |
| TC-07 | Obstacle during auto-closing | PASS |
| TC-08 | Obstacle overrides held CLOSE | PASS |
| TC-09 | Obstacle ignored during opening | PASS |
| TC-10 | Open Limit stops gate | PASS |
| TC-11 | Closed Limit stops gate | PASS |
| TC-12 | Wrong limit ignored | PASS |
| TC-13 | Security OPEN overrides Driver CLOSE | PASS |
| TC-14 | Security CLOSE overrides Driver OPEN | PASS |
| TC-15 | Driver panel conflict – stopped state | PASS |
| TC-16 | Driver panel conflict – moving state | PASS |
| TC-19 | Queue delivers events without loss | PASS |
| TC-20 | Mutex protects shared state | PASS |
| TC-21 | Semaphore signals limit activation | PASS |

Detailed Test Cases

TC-01 — Manual OPEN – stops on release   [PASS]

| Category | Manual Mode |
| --- | --- |
| Objective | Gate moves while OPEN held; stops on release before limit |
| Precondition | System running, state = IDLE_CLOSED |
| Steps | 1. Hold DrvOPN (PF4) > 400 ms 2. Observe GREEN LED ON 3. Release button before Open Limit |
| Expected | GREEN LED turns ON while held. On release → STOPPED_MIDWAY, all LEDs off (BLUE on). |
| Actual | GREEN LED ON while held. On release GREEN off, BLUE on -> STOPPED_MIDWAY |
| Serial Output | [Btn] DRV_OPEN pressed ¦ [Gate] OPENING ¦ [Btn] DRV_OPEN rel ms: 850 ¦ [Gate] Mode: MANUAL stop ¦ [Gate] STOPPED_MIDWAY |
| Notes | — |

Screenshot:

TC-02 — Manual CLOSE – stops on release   [PASS]

| Category | Manual Mode |
| --- | --- |
| Objective | Gate moves while CLOSE held; stops on release before limit |
| Precondition | State = IDLE_OPEN |
| Steps | 1. Hold DrvCLS (PE0) > 400 ms 2. Observe RED LED ON 3. Release before Closed Limit |
| Expected | RED LED ON while held. On release → STOPPED_MIDWAY, BLUE LED on. |
| Actual | RED LED ON while held. On release RED off, BLUE on -> STOPPED_MIDWAY |
| Serial Output | [Btn] DRV_CLOSE pressed ¦ [Gate] CLOSING ¦ [Btn] DRV_CLOSE rel ms: XXXX ¦ [Gate] Mode: MANUAL stop ¦ [Gate] STOPPED_MIDWAY |
| Notes | — |

Screenshot:

TC-03 — Manual from STOPPED_MIDWAY   [PASS]

| Category | Manual Mode |
| --- | --- |
| Objective | Manual control works in both directions from STOPPED_MIDWAY |
| Precondition | State = STOPPED_MIDWAY |
| Steps | 1. Hold DrvOPN > 400 ms, release → STOPPED_MIDWAY 2. Hold DrvCLS > 400 ms, release → STOPPED_MIDWAY |
| Expected | Gate responds in both directions. Stops each time button is released. |
| Actual | Gate opens and closes correctly from STOPPED_MIDWAY in manual mode, stops on release each time. |
| Serial Output | [Btn] DRV_OPEN pressed ¦ [Gate] OPENING ¦ [Gate] Mode: MANUAL stop ¦ [Gate] STOPPED_MIDWAY ¦ [Btn] DRV_CLOSE pressed ¦ [Gate] CLOSING ¦ [Gate] Mode: MANUAL stop ¦ [Gate] STOPPED_MIDWAY |
| Notes | — |

Screenshot:

TC-04 — Auto OPEN from IDLE_CLOSED   [PASS]

| Category | Auto Mode |
| --- | --- |
| Objective | Tap OPEN causes gate to move all the way until Open Limit pressed |
| Precondition | State = IDLE_CLOSED |
| Steps | 1. Tap DrvOPN < 400 ms 2. Observe GREEN LED stays ON after release 3. Press OPN LIM (PB1) |
| Expected | GREEN LED stays ON after tap. After OPN LIM → IDLE_OPEN, all LEDs off. |
| Actual | GREEN LED stays ON after tap. Gate moved to IDLE_OPEN after Open Limit pressed. |
| Serial Output | [Btn] DRV_OPEN pressed ¦ [Gate] OPENING ¦ [Btn] DRV_OPEN rel ms: <400 ¦ [Gate] Mode: AUTO ¦ [Input] Open Limit pressed ¦ [Gate] IDLE_OPEN |
| Notes | — |

Screenshot:

TC-05 — Auto CLOSE from IDLE_OPEN   [PASS]

| Category | Auto Mode |
| --- | --- |
| Objective | Tap CLOSE causes gate to move until Closed Limit pressed |
| Precondition | State = IDLE_OPEN |
| Steps | 1. Tap DrvCLS < 400 ms 2. GREEN LED should be off, RED LED ON 3. Press CLS LIM (PD0) |
| Expected | RED LED stays ON after tap. After CLS LIM → IDLE_CLOSED, all LEDs off. |
| Actual | RED LED stays ON after tap. Gate moved to IDLE_CLOSED after Closed Limit pressed. |
| Serial Output | [Btn] DRV_CLOSE pressed ¦ [Gate] CLOSING ¦ [Btn] DRV_CLOSE rel ms: <400 ¦ [Gate] Mode: AUTO ¦ [Input] Closed Limit pressed ¦ [Gate] IDLE_CLOSED |
| Notes | — |

Screenshot:

TC-06 — Auto mode from STOPPED_MIDWAY   [PASS]

| Category | Auto Mode |
| --- | --- |
| Objective | Auto mode works from mid-position |
| Precondition | State = STOPPED_MIDWAY |
| Steps | 1. Tap DrvOPN < 400 ms → GREEN stays on 2. Press OPN LIM → IDLE_OPEN 3. Reset to STOPPED_MIDWAY 4. Tap DrvCLS < 400 ms → RED stays on 5. Press CLS LIM → IDLE_CLOSED |
| Expected | Both directions work from STOPPED_MIDWAY in auto mode. |
| Actual | Auto mode works from STOPPED_MIDWAY in both directions. |
| Serial Output | [Btn] DRV_OPEN pressed ¦ [Gate] OPENING ¦ [Gate] Mode: AUTO ¦ [Input] Open Limit pressed ¦ [Gate] IDLE_OPEN ¦ [Btn] DRV_CLOSE pressed ¦ [Gate] CLOSING ¦ [Gate] Mode: AUTO ¦ [Input] Closed Limit pressed ¦ [Gate] IDLE_CLOSED |
| Notes | — |

Screenshot:

TC-07 — Obstacle during auto-closing   [PASS]

| Category | Obstacle |
| --- | --- |
| Objective | Obstacle triggers stop → reverse 500ms → STOPPED_MIDWAY |
| Precondition | State = CLOSING (RED LED ON, auto mode) |
| Steps | 1. Tap DrvCLS → CLOSING 2. Press OBSTACLE (PD1) 3. Observe: RED off, GREEN+BLUE blink ~500ms 4. After 500ms observe BLUE solid |
| Expected | Immediate stop, reverse (GREEN+BLUE blink 500ms), then STOPPED_MIDWAY (BLUE solid). |
| Actual | Obstacle triggered reversal: RED off, GREEN+BLUE blink 500ms, then STOPPED_MIDWAY. |
| Serial Output | [Btn] DRV_CLOSE pressed ¦ [Gate] CLOSING ¦ [Gate] Mode: AUTO ¦ [Input] OBSTACLE pressed ¦ [Safety] OBSTACLE! Reversing... ¦ [Gate] REVERSING ¦ [Safety] Reverse done ¦ [Gate] STOPPED_MIDWAY |
| Notes | — |

Screenshot:

TC-08 — Obstacle overrides held CLOSE   [PASS]

| Category | Obstacle |
| --- | --- |
| Objective | Safety overrides even a manually held CLOSE button |
| Precondition | State = CLOSING (RED LED ON, manual mode – button held) |
| Steps | 1. Hold DrvCLS (manual mode) 2. While RED LED ON, press OBSTACLE 3. Observe reversal behavior |
| Expected | Same as TC-07 – obstacle overrides the held button. |
| Actual | Safety override works even with CLOSE held manually. Gate reversed and stopped. |
| Serial Output | [Btn] DRV_CLOSE pressed ¦ [Gate] CLOSING ¦ [Input] OBSTACLE pressed ¦ [Safety] OBSTACLE! Reversing... ¦ [Gate] REVERSING ¦ [Safety] Reverse done ¦ [Gate] STOPPED_MIDWAY |
| Notes | — |

Screenshot:

TC-09 — Obstacle ignored during opening   [PASS]

| Category | Obstacle |
| --- | --- |
| Objective | Obstacle has no effect while gate is OPENING |
| Precondition | State = OPENING (GREEN LED ON) |
| Steps | 1. Tap DrvOPN → OPENING 2. Press OBSTACLE 3. Observe GREEN LED |
| Expected | GREEN LED stays ON, gate continues opening. Obstacle is ignored. |
| Actual | Obstacle ignored during OPENING. GREEN LED stayed ON, gate continued opening. |
| Serial Output | [Btn] DRV_OPEN pressed ¦ [Gate] OPENING ¦ [Input] OBSTACLE pressed ¦ [Safety] Obstacle ignored ¦ GREEN LED remains ON |
| Notes | — |

Screenshot:

TC-10 — Open Limit stops gate   [PASS]

| Category | Limit Buttons |
| --- | --- |
| Objective | Open Limit button transitions OPENING → IDLE_OPEN |
| Precondition | State = OPENING (GREEN LED ON) |
| Steps | 1. Tap DrvOPN → OPENING 2. Press OPN LIM (PB1) |
| Expected | GREEN LED turns OFF → IDLE_OPEN, all LEDs off. |
| Actual | Open Limit pressed during OPENING -> IDLE_OPEN. GREEN LED turned off. |
| Serial Output | [Btn] DRV_OPEN pressed ¦ [Gate] OPENING ¦ [Input] Open Limit pressed ¦ [Gate] Open limit hit ¦ [Gate] IDLE_OPEN |
| Notes | — |

Screenshot:

TC-11 — Closed Limit stops gate   [PASS]

| Category | Limit Buttons |
| --- | --- |
| Objective | Closed Limit transitions CLOSING → IDLE_CLOSED |
| Precondition | State = CLOSING (RED LED ON) |
| Steps | 1. Tap DrvCLS → CLOSING 2. Press CLS LIM (PD0) |
| Expected | RED LED turns OFF → IDLE_CLOSED, all LEDs off. |
| Actual | Closed Limit pressed during CLOSING -> IDLE_CLOSED. RED LED turned off. |
| Serial Output | [Btn] DRV_CLOSE pressed ¦ [Gate] CLOSING ¦ [Input] Closed Limit pressed ¦ [Gate] Closed limit hit ¦ [Gate] IDLE_CLOSED |
| Notes | — |

Screenshot:

TC-12 — Wrong limit ignored   [PASS]

| Category | Limit Buttons |
| --- | --- |
| Objective | Pressing wrong limit while moving has no effect |
| Precondition | State = OPENING (GREEN LED ON) |
| Steps | 1. Tap DrvOPN → OPENING 2. Press CLS LIM (PD0) – wrong limit |
| Expected | GREEN LED stays ON, gate continues opening. No state change. |
| Actual | Closed Limit pressed during OPENING had no effect. Gate continued opening. |
| Serial Output | [Btn] DRV_OPEN pressed ¦ [Gate] OPENING ¦ [Gate] Wrong limit ignored ¦ GREEN LED remains ON |
| Notes | — |

Screenshot:

TC-13 — Security OPEN overrides Driver CLOSE   [PASS]

| Category | Security Priority |
| --- | --- |
| Objective | Security command wins when both panels active |
| Precondition | State = IDLE_CLOSED or CLOSING |
| Steps | 1. Hold DrvCLS (Driver CLOSE active) 2. Simultaneously press SecOPN (Security OPEN) 3. Observe LEDs |
| Expected | GREEN LED turns ON – gate opens. Security command overrides driver. |
| Actual | Security OPEN overrides Driver CLOSE. Gate opened (GREEN on). |
| Serial Output | [Btn] DRV_CLOSE pressed ¦ [Gate] CLOSING ¦ [Btn] SEC_OPEN pressed ¦ [Gate] Security owns CLOSING ¦ [Gate] OPENING |
| Notes | — |

Screenshot:

TC-14 — Security CLOSE overrides Driver OPEN   [PASS]

| Category | Security Priority |
| --- | --- |
| Objective | Security CLOSE wins over Driver OPEN |
| Precondition | State = IDLE_OPEN or OPENING |
| Steps | 1. Hold DrvOPN (Driver OPEN active) 2. Simultaneously press SecCLS (Security CLOSE) 3. Observe LEDs |
| Expected | RED LED turns ON – gate closes. Security command overrides driver. |
| Actual | Security CLOSE overrides Driver OPEN. Gate closed (RED on). |
| Serial Output | [Btn] DRV_OPEN pressed ¦ [Gate] OPENING ¦ [Btn] SEC_CLOSE pressed ¦ [Gate] CLOSING |
| Notes | — |

Screenshot:

TC-15 — Driver panel conflict – stopped state   [PASS]

| Category | Conflict |
| --- | --- |
| Objective | Simultaneous OPEN+CLOSE on driver panel → safe stop |
| Precondition | State = IDLE_CLOSED or STOPPED_MIDWAY |
| Steps | 1. Press DrvOPN and DrvCLS simultaneously |
| Expected | No motion. BLUE LED on → STOPPED_MIDWAY. |
| Actual | Driver OPEN+CLOSE simultaneously -> STOPPED_MIDWAY. BLUE LED on. |
| Serial Output | [Input] Driver CONFLICT ¦ [Gate] CONFLICT -> STOP ¦ [Gate] STOPPED_MIDWAY |
| Notes | — |

Screenshot:

TC-16 — Driver panel conflict – moving state   [PASS]

| Category | Conflict |
| --- | --- |
| Objective | Simultaneous OPEN+CLOSE while gate moving → stops safely |
| Precondition | State = OPENING or CLOSING |
| Steps | 1. Start gate moving (tap DrvOPN → OPENING) 2. Press DrvCLS while still moving |
| Expected | Gate stops → STOPPED_MIDWAY. BLUE LED on. |
| Actual | Driver OPEN+CLOSE while moving -> STOPPED_MIDWAY. Gate stopped safely. |
| Serial Output | [Gate] OPENING ¦ [Input] Driver CONFLICT ¦ [Gate] CONFLICT -> STOP ¦ [Gate] STOPPED_MIDWAY |
| Notes | — |

Screenshot:

TC-19 — Queue delivers events without loss   [PASS]

| Category | RTOS Mechanisms |
| --- | --- |
| Objective | Rapid button presses are all received by Gate Control |
| Precondition | System running |
| Steps | 1. Press multiple buttons rapidly in succession 2. Observe serial output – each press should be logged |
| Expected | Serial window shows every button press logged with no missing events. |
| Actual | All rapid button presses logged in serial window. No events dropped. |
| Serial Output | All [Btn] press events appear in serial output with no gaps. |
| Notes | — |

Screenshot:

TC-20 — Mutex protects shared state   [PASS]

| Category | RTOS Mechanisms |
| --- | --- |
| Objective | g_gate_state is never corrupted under concurrent access |
| Precondition | System running normally |
| Steps | 1. Trigger obstacle during closing (Safety Task writes state) 2. Simultaneously trigger button events (Gate Control reads state) 3. Observe correct behavior with no corruption |
| Expected | State transitions are always valid. No undefined states observed. |
| Actual | No state corruption observed. All transitions valid during concurrent obstacle + button events. |
| Serial Output | State transitions remained valid. Mutex protected shared state correctly. |
| Notes | — |

Screenshot:

TC-21 — Semaphore signals limit activation   [PASS]

| Category | RTOS Mechanisms |
| --- | --- |
| Objective | xSemLimit semaphore is given on limit press and consumed correctly |
| Precondition | State = OPENING |
| Steps | 1. Start auto-opening 2. Press OPN LIM → semaphore given by Input Task 3. Gate Control receives it and stops |
| Expected | Gate stops immediately at IDLE_OPEN. Semaphore mechanism works alongside queued event. |
| Actual | Gate stopped immediately at IDLE_OPEN. Semaphore signalled by Input Task consumed by Gate Control. |
| Serial Output | [Input] Open Limit pressed ¦ [Gate] Open limit hit ¦ [Gate] IDLE_OPEN |
| Notes | — |

Screenshot: